# Material Safety Data Sheet

May be used to comply with

OSHA’s Hazard Communication Standard

29 CFR 1910.1200.  Standard must be

consulted for specific requirements.

## Product Identity – Section I

# Product Name:   RUST OFF

CAS #:   See Below

Manufacturer:   Arcal Chemicals, Inc.

# Address:          223 Westhampton Avenue

Capitol Heights, MD  20743

Emergency Tele:  (800) 424-9300

Revision Date:  01 January 2014

Other Tele:  (301) 336-9300

# Fax:  (301) 336-6597

## HAZARDOUS INGREDIENTS / OTHER COMPONENTS – SECTION II

### MATERIAL / CAS #

ACGIH (TLV – TWA)

OSHA (PEL -TWA)

Hydrogen Chloride	CAS # 7647-01-1

Phosphoric Acid	CAS # 7664-38-2

Ammonium Bifluoride	CAS # 1341-49-7

5 PPM

1 mg/m3

2.5 mg/m3

5 PPM

1 mg/m3

2.5 mg/m3

#### ODOR:  Acidic, choking

#### MELT POINT:  N/A

#### APPEARANCE :  Clear solution

SPECIFIC GRAVITY:

1.22

% VOLATILE BY VOLUME:  N/A

#### BULK DENSITY:  N/A

#### VAPOR DENSITY:  N/A

#### BOILING POINT:  212F

% SOLUBILITY (H20):           	Complete

#### VAPOR PRESSURE: N/A

## PHYSICAL DATA – SECTION III

### FIRE AND EXPLOSION HAZARD DATA – SECTION IV

HEALTH

FLAMMABILITY

REACTIVITY

SPECIFIC

### NFPA FIRE HAZARDS

3

0

0

FLASH POINT:  None

FLAMMABLE LIMITS             LEL:  None				UEL:  None

EXTINGUISHING MEDIA:  CO2  and water.  This product will not support combustion.

SPECIAL FIRE FIGHTING PROCEDURES:  Use self-contained breathing apparatus.  Keep container cool.  Ventilate area as much as possible.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Acidic product.  Reacts with metals to produce hydrogen.

DECOMPOSITION PRODUCTS UNDER FIRE CONDITIONS: CO2, and oxides of nitrogen with incomplete combustion; hydrogen fluoride gas when heated.

EMPTY CONTAINER WARNING: Empty containers retain hazardous residue.   Keep away from alkaline material, metals and oxidizing agents.  Rinse container before discarding.

4 = EXTREME

3 = HIGH

2 = MODERATE

1 = SLIGHT

0 = INSIGNIFICANT

### REACTIVITY DATA – SECTION V				              Rust Off

Stability:  Stable & polymerization will not occur			CONDITIONS TO AVOID:  None known

INCOMPATIBILITY (MATERIAL TO AVOID):  Avoid contact with alkaline material, metal powders and strong oxidizing agents.

HAZARDOUS DECOMPOSITION OR BYPRODUCTS: CO2, and oxides of nitrogen with incomplete combustion.

HAZARDOUS POLYMERZATION:	Will not occur			CONDITIONS TO AVOID:  None known

## HEALTH HAZARD DATA – SECTION VI

Route(s) of Entry:	Inhalation?	Yes		Skin?	Yes		      Ingestion?    Yes

Health Hazards (Acute and Chronic):  Irritation or burns on skin and eyes; irritation of mucous membranes.  Irritation of respiratory tract with burning and coughing.  Fluoride will penetrate skin and can cause serious internal damage, which will be apparent only hours later.

CARCINOGENICITY:  None			NTP?	N/A		IARC Monographs?	N/A	OSHA? N/A

MEDICAL CONDITIONS

GENERALLY AGGRAVATED by EXPOSURE:  Skin and eye irritation, dizziness, respiratory tract irritation and other lung conditions.

##### EMERGENCY AND FIRST AID PROCEDURES

EYE CONTACT:  If splashed into eyes, rinse eyes with large amounts of water for at least 15 minutes or until irritation subsides, and get immediate medical attention.

SKIN CONTACT:  In case of skin contact, remove any contaminated clothing, and flush skin with cold water for 15 minutes; soak contaminated area in 0.13% iced aqueous Zephiran Chloride solution for 30-60 minutes and call a physician immediately.

INHALATION:  If overcome by vapor, remove person to fresh air if breathing is difficult, and give oxygen; seek medical attention.   If breathing has stopped, start resuscitation, and administer oxygen if available.  Seek immediate medical attention.

INGESTION:  If ingested, DO NOT induce vomiting; call a physician immediately.

### PRECAUTIONS FOR SAFE HANDLING AND USE – SECTION VII

Steps to be taken in case material is released or spilled:  Contain spill.  Neutralize with hydrated lime.  Flush area with water.

Waste Disposal Method: Adjust pH and dispose in accordance with federal, state and local regulations.

Precautions to be taken in handling and storing:  Store in cool, well-ventilated area.  Keep containers closed when not in use.  Separate from oxidizing agents and alkalis.  Avoid skin contact; remove contaminated clothing.

### CONTROL MEASURES – SECTION VIII

Respiratory or breathing apparatus:  Respirator rated for acid fumes.

Ventilation:  Yes		Local Exhaust:					Special:

Mechanical:  General air dilution			Other:

Protective Gloves:  Chemical resistant/rubber		Eye Protection:  Face shield or goggles

Other Protective Clothing or Equipment:  Chemical resistant apron, boots and other protective gear.

###### TRANSPORTATION INFORMATION

UN2922, CORROSIVE LIQUIDS, TOXIC N.O.S. (HYDROGEN CHLORIDE, AMMONIUM BIFLUORIDE), 8 (6.1), PG:II