__________________________________________________________________________________________

Toll Free: (888) 833-8308								Durajoint Concrete Accessories

Fax: (330) 527-2440 										10421 Industrial Drive

Web: www.durajoint.com									Garrettsville, OH 44231

Retro-Fit Waterstop

Type 29 Kit

Weight: (lbs/ft) .82			1 – 10 ft. piece of Type 29

Width: 3”				1 – 10 ft. Stainless Baton Bar___

Thickness: 3/16” 			10 – Stainless Steel Bolts & Epoxy

1/4”

1/2”

12”

3/16”

7/8 ID & 1-1/2” OD

Installation:

- The old concrete surface must be dry and clean. To achieve the proper surface preparation sandblasting and/or grinding will be required to insure adhesion of the attachment system.

- Apply a bed of W.R. Meadows Rezi-Weld Gel Paste Epoxy completely covering the prepared surface with a notched trowel approximately 1/8” wider than the retrofit waterstop base.

- Place the retrofit waterstop base against the bed of epoxy.

- Begin securing the retrofit waterstop base by drilling holes through the retrofit waterstop base into the existing concrete.  Make sure the holes matches the stain steel baton bar hole pattern.

- Slide the provided self tapping screw through the pre-drilled baton bar hole and into the previous drilled base/concrete hole. Attach the top side of the retrofit waterstop base first, than attach the bottom.

Waterstop • Forming Accessories • Rebar Supports • Mesh Supports • Prestressed Accessories